Index

1. Purpose

2. Scope

3. Process Owner

4. ISO9001:2015 Ref. Clause

5. Definitions & Abbreviations

6. Process Flow

7. Procedural Steps

8. Stakeholders

Revision History

| Issue # | Issue Date | Rev. # | Rev. Date | Description of Change | Section Affected |
| --- | --- | --- | --- | --- | --- |
| 03 | 29-Feb-2020 | 01 | 29-Feb-2020 | Added DOs and DONTs, maintenance etc. | NA |
| 02 | 01-Jun-2018 | 00 | 16-Aug-2018 | Issue 02 as per ISO 9001:2015 & changes done as per new revised procedure format | NA |

1. Purpose: To standardize software / firmware coding style across all software development activity in SW function.

2. Scope: All in house developed software and firmware. I.e. all new C code that we write shall follow this standard. This standard shall be also followed while modifying legacy code, see Section 5 Legacy Code for more details.

3. Process Owner: SW

4. ISO9001:2015 Ref. Clause: 8.3.4

5. Definitions & Abbreviations: NA

6. Process Flow: NA

5. Procedural Steps: Described in the table below.

| Sr. No. | Activity | Resp. | Doc. Ref. |
| --- | --- | --- | --- |
| 1 | While writing software or firmware for any automotive, off-highway or other application: always follow the coding standard as described in detail in the sections below. | Software development team members – including Engineer and Lead Engineer. | None. |

Table of Contents

1 Introduction 3

2 Naming convention 3

2.1. Description 3

2.2. Data types: 3

2.3. Variable 4

2.4. Function 5

2.5. File 5

2.6. Macro 6

2.7. Struct & Enum 7

3. Indentation & Spacing Style 7

3.1. Description 7

3.2. If-Else 7

3.3. For loop 8

3.4. While loop 8

3.5. Switch statement 9

3.6. #if Macros 9

3.7. Comments 10

3.8. Enum 10

3.9. Private Function 10

3.10. Public API Function 11

3.11. Module 11

3.12. File structure 11

4. Other DOs and DONTs: 12

4.1. Minimal use of extern variables 12

4.2. No compiler warnings 12

4.3. Limit sizes of functions 12

4.4. Limit complexity 12

5. Legacy Code 12

5.1. Modifying to match this standard: 12

5.2. Maintenance: 12

- Introduction:

This standard was born in an effort to standardize software / firmware-coding style across all software development activity within SW function in SEDEMAC. Salient features of standard are:

- Camel Case naming convention for functions/variables etc. & snake case naming for files & folders.

- Minimal Apps Hungarian Notation specifying "semantic type" of entities

- A .h file for (almost) every .c file

- No usage of tabs (\t). Instead, 1 indent = 4 spaces

A style guide has a two-fold purpose:

- Help in collaboration between multiple developers (also between past & present instances of the same developer) by maintaining uniformity.

- Code beautification. As programmers, we keep looking at code all day long. A good coding style guide helps ensure just that.

The SW coding standard at SEDEMAC has been designed keeping these two design rules in mind.

Apart from this coding standard, it is recommended to follow guidelines from this page.

- Naming convention:

## 2.1.  Description:

As mentioned previously, the standard uses the Apps Hungarian notation for variable naming. Shown below is the list of prefixes.

2.1.1 Prefixes that indicate scope, pointer / non-pointer etc.

| Prefix | Scope, pointer or non-pointer etc |
| --- | --- |
| g | prefixed to global variables |
| p | prefixed to pointer variable |
| fp | prefixed to function pointers |
| n | [Optionally] prefixed to variables intended to be used as counters |
| e | prefixed to enum variables |

Variables or functions which satisfy multiple prefix conditions should contain all valid prefixes, with g, p, fp having a higher priority than the rest of the prefixes.

2.1.2 Prefixes that indicate data type and / or size of variable:

| Prefix | Data type and / or size |
| --- | --- |
| u8, u16, u32, u64 | Unsigned integer variables of sizes 8, 16, 32, 64 bits, respectively. |
| i8, i16, i32, i64 | Signed integer variables of sizes 8, 16, 32, 64 bits, respectively. |
| b | bool type variable |
| f32, f64 | Floating point variables of sizes 32 or 64 bits, respectively. |

These prefixes should be applied after the prefixes that indicate scope etc. Some examples of valid variable names with combinations of prefixes:

uint32_t gu32TimeFromAdvToZero_ms;	// global variable

uint8_t u8Data;		// local variable or function parameter

bool gbEngineOn = false;	// global

float *gpf32Multipliers;	// global pointer variable

GOVERNING_MODE eGoverningMode;	// function parameter

2.1.3 Variables with C standard integral data types

Sometimes we declare variables of standard integral data types instead of size specific types (as recommended in the page “Data types: smaller may not be always better”). In such cases, it is recommended that we use prefixes i or u to indicate int or unsigned int data type, respectively. In this case, it is also okay to not use any prefix at all, but instead start the variable name with the letter i or u, to indicate int or unsigned int data type, respectively. E.g. i, u, i1, i2, iTemp, uTemp, u1, u2 are acceptable variable names only if scope of the variable is limited to 30 lines of code or less. Using data types long int, unsigned long int, short int, unsigned short int, long long int, unsigned long long int is not recommended.

## 2.2. Data types:

MISRA C recommends always using size-specific data types (such as uint8_t, int32_t etc.). For all firmware and possibly also for some other applications: we shall follow this recommendation, barring some exceptions as mentioned on this page.

## 2.3. Variable:

- {types}{VariableName}_{units}, where the variable of interest signifies a physical entity such as mass, velocity, voltage.

int16_t Vbat_mV;                    // millivolt

uin16_t gTimeSinceInterrupt_uSec;   // microsecond

- {types}{VariableName}, where there are no specific units for the variable of interest.

SYSTEM_STATE_ gSystemState;    // structure containing system state

uint8_t nOverflows;            // number of overflows

- Prefixes: that indicate scope, data type or the intended usage of a variable maybe optionally used for variable names. Some examples of good variable names:

- int16_t i16Vbat_mV;

- // i16 prefix indicates int16_t data type type.

- // i8, i16, i32, i64, u8, u16, u32, u64 prefixes for respective data

- // types (signed or unsigned integer types of various sizes).

- // b prefix to indicate boolean variable.

- uint16_t nOverflows; // n prefix indicates it is a “count”

- // prefix “g” shall be added to indicate global scope.

- // further, prefix “g” shall be the first prefix (if there are more

- // than one prefixes). e.g.

- uint32_t gu32EngineRpm = 0;

- bool gbCrankingAllowed = false;

- Variables which have a well defined local scope may have naming convention as {variablename}.

int8_t now, prev, tmp;

- Occasionally, this convention can be broken for cases such as iteration index in for() loops (eg: i, j, k)

- While declaring strings, the term char should be used instead of int8_t to explicitly specify intention of the variable. In other cases, when the variable is intended to be used as an 8 bit signed number, int8_t should be used

char* pcVersionName;         // version name (pointer to string)

- Arrays should not have p prefix. Only variables explicitly declared as a pointer should have p prefix

uint8_t RxBuffer[10];

uint8_t* pRxBufferHead;

- Double pointers will have pp prefix (basically one p for every level of pointer)

int8_t** ppStackAddr;

- Calibration variables should begin with a CAL_ prefix

int8_t CAL_IgnTimingMap[46][11];

uint16_t CAL_CRANKING_FWD_REGULATION_RPM;

- Single dimensional calibration tables should have Curve as a suffix. Two dimensional calibration tables should have Map as a suffix. The motivation behind this convention draws from a similar naming convention used by calibration tools such as Vector CANape. Three dimensional maps should have Map3D as a suffix (MapND for N dimensional table for N greater than 2)

int16_t CAL_LiCurve[60];

int8_t CAL_IgnTimingMap[46][11];

int8_t CAL_RegulationMap3D[10][10][10];

- Arrays used as a temporary storage mechanism (such as those for CAN messages_ should have Buffer as suffix

uint16_t RxBuffer[10];

## 2.3.1 Standard suffixes to indicate units of measurement

| Suffix | Unit of measurement |
| --- | --- |
| _sec | Seconds |
| _ms | Milli-seconds |
| _tms | Tens of milli-seconds |
| _hms | Hundreds of milli-seconds |
| _us | Micro-seconds |
| _tus | Tens of micro-seconds |
| _hus | Hundreds of micro-seconds |
| _ns | Nano-seconds |
| _tns | Tens of nano-seconds |
| _hns | Hundreds of nano-seconds |
| _mV | Milli-Volts |
| _V | Volts |
| _degC | Degree Celsius |
| _degK | Degree Kelvin |
| _L | Litres |
| _mL | Milli-litres |
| _m | Meters |
| _km | Kilometres |
| _cm | Centimetres |
| _mm | Millimetres |
| _rpm | RPM |

  - Function :

- {FunctionName}_{units}, where the function returns either a physical unit or a computational unit. While returning raw ADC counts, the suffix raw should be used

int16_t GetVbat_mV(void);

int16_t GetVbat_raw(void);

- {FunctionName}, where the return type does not have physical or computational unit

void SetHeadlampPwmDuty(uint8_t duty);

ERROR_CODE_t UpdateTask(TASK_t* pTask);

int16_t* FetchAddress(uint8_t index);

void Interp1D(CURVE_t* pCurve, int16_t x);

- All private functions (static scope) should begin with prv.

int32_t prvConvertAdcToVoltage(void);

- For mathematical functions used in common libraries, it is preferred to add suffixes indicating input type, and prefixes indicating output type in the function name. Note that if the mathematical function is defined in local scope and there is no ambiguity in the input/output types, then the type information from the function name may be dropped to improve readabilty.

int32_t S32_Interp1D_S16(CURVE_t* pCurve, int16_t value);

uint16_t U16_Mul_U8_U8(uint8_t value1, uint8_t value2);

  - File :

- .c & .h files follow snake case convention with small letters. {file_name}.c & {file_name}.h.

ignition_scheduler.c

series_regulation.c

hw.c

config.h

- .md (Markdown), .txt files follow snake case convention with capital letters {FILE_NAME}.md, {FILE_NAME}.txt

README.md

LICENSE.md

RELEASE_NOTES.md

CHANGELOG.txt

- Folders follow snake case convention, either as {folder_name} or {FOLDER_NAME}. Capital letter convention should be used sparingly; only to be invoked for dealing with proper nouns

isg_states

PCBP73_R5F10BMG

- Calibratable variables should be divided amongst two files: sed_cal.c, oem_cal.c. These files should contain variables which begin with CAL_ prefix

- Each .h file must have macro safeguards with the naming convention as FILE_NAME_H

// safeguards for ignition_logic.h

#ifndef IGNITION_LOGIC_H

#define IGNITION_LOGIC_H

#endif

- If a compiler does not implement definitions of standard types such as int8_t and the likes, the definitions of such files should be present in a user created file with the name of the header file having format: stdint_{microcontroller_family}.h. Example, stdint_rl78_f13.h

- In case a bunch of variables are to be externed across modules which have a client-vendor relationship, the extern definitions may be included in .h files. However, variables which are private from the point of view a specific logic implementation need not expose externed variables in header files. As an example, if a specific functionality is being implemented in a.c, b.c with the API definitions declared in c.h; and if a.c and b.c need to share a variable, then the variable extern statement should directly be added in .c files rather than in c.h. Only if the variables are a part of the interface provided by a functionality, should the extern statements be declared in .h files.

  - Macro:

- {category}{MACRO_NAME}, where category is optional and should be used whenever deemed appropriate

#define BATTERY_VOLTAGE_GAIN      (15)

#define DEBUG_MODE_ENABLED        (1)

#define configIGNITION_KEY_RISING (1)   // config "category"

#define ccBOOTLOADER_ENABLED      (1)   // settings "category"

- For mathematical operations:

#define SQUARE(X) (X*X)

- While using #ifdef #endif macros, a comment should be added in front of #endif to indicate which macro name the #endif belongs to

#ifdef DEBUG_MODE_ENABLED

.

.

.

#endif /* DEBUG_MODE_ENABLED */

  - Struct & Enum:

- Enum should always be typedefed & enum entries should be capital lettered. The enum type itself should be named as in {ENUM_TYPE}_t

typedef enum

{

EQUAL=0,

GREATER=1,

LESS=2,

} COMPARISON_t;

- Struct should always be typedefed & named as in {STRUCT_NAME}_t

typedef struct

{

int16_t Start;

uint8_t Length;

uint8_t Step;

int16_t* pHead;

} MAP1D_t;

- Indentation & Spacing Style:

## 3.1Description:

Tabs (\t) is not to be used at all in the code base, mainly because different editors render tabs differently (as equivalent of either 2 space, 4 space or 8 space). To avoid this confusion, 4 spaces should be used for each level of indentation.

## 3.2. If-Else:

- No space between if and (. Similarly, no spacing for else if and else lines

- { & } belong to their dedicated separate lines

- Operands in the condition block should be separated by a single space, example (first < second) as shown below

uint8_t first = 16, second = 51;

COMPARISON_t eFirstVsSecond;

if(first == second)

{

eFirstVsSecond = EQUAL;

}

else if(first > second)

{

eFirstVsSecond = GREATER;

}

else

{

eFirstVsSecond = LESS;

}

## 3.3. For loop:

- No space between for and (

- { & } belong to their dedicated separate lines

- All operands within round brackets should be separated by space from the binary operator. For example i <= usN

- Helper variables i, j, k can be used to iterate in for loops.

uint16_t Fibonacci(uint8_t N)

{

uint16_t prev = 0, now = 0, tmp = 0;

for(i = 0; i <= N; i++)

{

tmp = now;

now = now + prev;

prev = tmp;

}

return now;

}

  - While loop:

- Spacing rules are similar to if-else and for loop rules.

uint16_t Fibonacci(uint8_t N)

{

uint16_t prev = 0, now = 0, tmp = 0;

uint8_t i = 0;

while(i != N)

{

tmp = now;

now = now + prev;

prev = tmp;

i += 1;

}

return now;

}

  - Switch statement:

- default case should always be added.

switch(index)

{

case 1:

{

y = 13;

break;

}

case 2:

{

y = 27;

break;

}

default:

{

y = 0;

break;

}

}

  - #if Macros:

- Macros should be aligned with the first column as shown:

int main(void)

{

uint16_t i = 0, total = 0;

for (i = 0; i < 100; i++)

{

total = total + i;

#ifdef DEBUG_MODE_ENABLED

printf("%d", i)

#endif

}

return 0;

}

  - Comments:

- Comments using // should be avoided as much as possible.

- Comments shall be accurate. Inaccurate or outdated comments are worse than not having comments at all.

- All non-obvious functions/variables/macros should be commented using /* */

/* Returns n^th fibonacci number

If return value goes beyond 2^16, it is clamped at 2^16 */

uint16_t Fibonacci(uint8_t n);

/* This is the only instance of SystemState_t throughout the code.

Is updated every 1ms */

SYSTEM_STATE_t gSystemState;

/* Enables printing debug strings over UART from printf */

#define DEBUG_MODE_ENABLED  (1)

- If a block of code is commented out: an additional comment above the block shall explain:

  - Who commented the code block?

  - When?

  - Why?

  - Will this code block need to be ever un-commented? If yes, when and why? If not, why this block is just commented rather than deleted?

  - Conditional compiling directives:

- Conditional compiling maybe used for:

  - implementing optional features

  - creating variants

- Preferable way for conditional compiling is to use #if rather than #ifdef.

Good example:

#define MK20FX512VLQ12 20

#define MK60FN1M0VLQ12 60

#define MCU_USED MK20FX512VLQ12

#if (MCU_USED == MK20FX512VLQ12)

// write code specific to MK20FX512VLQ12

#else if (MCU_USED == MK60FN1M0VLQ12)

// write code specific to MK60FN1M0VLQ12

#else

// no valid MCU is selected, so throw error message:

#error MCU_USED can only be either MK20FX512VLQ12 or MK60FN1M0VLQ12

#endif

Bad example:

#define MK60 1

#ifdef MK60

// write code specific to MK60

#else

// write code specific to MK20

#endif

  - Enum:

/* Defines the "broad" vehicle level state from ISG perspective */

typedef enum

{

VEHICLE_STATE_IGNOFF=0,      /* Ignition key OFF */

VEHICLE_STATE_IDLE_STOP=1,   /* Idle start stop mode */

VEHICLE_STATE_RUNNING=2,     /* Ignition ON & charging */

VEHICLE_STATE_CRANKING=3,    /* Cranking */

VEHICLE_STATE_STOPPING=4,    /* Repositioning engine */

VEHICLE_STATE_UNKNOWN=255,   /* None of the above */

} VEHICLE_STATE_t;

  - Private Function:

/**************************************************************************

Function: prvReadFrontWheelSpeed

Description: The front wheel speed sensor is a variable frequency sensor which

generates a 0-2500Hz square wave at 50% duty cycle. The value returned by

this API is counts corresponding to the time period of the sensor output.

To convert the counts into seconds, the function software should multiply the

returned value by 1e-6.

Input parameters: None

Output parameters: None

Return value: None

**************************************************************************/

static void prvReadFrontWheelSpeed(void);

## 3.11. Public API Function:

/**************************************************************************

Function: GetVehicleState

Description: This API returns the current vehicle state.

Input parameters: None

Output parameters: None

Return value: Current vehicle state; encoded as documented in VEHICLE_STATE_t

**************************************************************************/

VEHICLE_STATE_t GetVehicleState(void);

## 3.12. Module:

/**************************************************************************

* Copyright (C) 2016, SEDEMAC Mechatronics Pvt. Ltd. All rights reserved.

*

* Module name: bsp.c

* Version: 2.3 (optional)

* Author: Ajit Natu

* Date created: 30.06.2016

* External references: None.

*

* Module description:

* This module implements the APIs exported to the Function software.

* The APIs allow the function software to read sensor inputs and actuate

* the outputs without being aware of the low level ECU hardware details.

* Change history

* {Date}: {Developer}: {Changes}

* {Date}: {Developer}: {Changes}

* {Date}: {Developer}: {Changes}

**************************************************************************/

- In case the module implements common library functions, (such as interpolation), then the module should have a version number

- In case of change request on a stable module, change history should be added to the module comments

## 3.13. File structure:

- C file

  - Module header

  - #include statements (must at least include .h file of this module)

  - Macro definitions

  - Private declarations

  - Private function declarations

  - Public function definitions

  - Private function definitions

- H file

  - Module header (same as corresponding C file)

  - #ifndef statement

  - #include statements

  - Macro definitions

  - Public function declarations

  - extern variable declarations

# 4. Other DOs and DONTs:

## 4.1. Minimal use of extern variables:

Each module should define it’s own global variables. Most of these may not need to be accessed directly by other modules, these should be defined as static. If at all some variables need to be directly accessed by other modules: then such variables should be declared as extern in the module’s header file. Each module’s C file shall include it’s own H file. As far as possible, implement get and set functions rather than allowing other modules to directly access global variables.

## 4.2. No compiler warnings:

Compiler setting shall be “enable all warnings”. All warnings shall be removed or explicitly justified if they cannot be removed. This “justification” shall be in the form of a comment, immediately after the line that causes this warning.

## 4.3. Limit sizes of functions:

Each function shall be sufficiently small for easy understanding. Ideal is <= 50 lines.

## 4.4. Limit complexity:

Cyclomatic complexity of each function shall be low – ideally <= 15.

## 4.5. Use size specific data types:

Standard size specific data types shall be used – e.g. int8_t, uint32_t etc. Since these are defined in the header file <stdint.h>, it must be #included in every C file. In case your compiler does not provide this file, you may copy the header file from gcc and modify it’s contents to suit the architecture of your microcontroller. Alternatively, you may create your own version of this file, specific to the microcontroller and compiler you are using. One such example file is stdint_rl78.h – which is good for Renesas RL78 microcontroller family. You may copy it and modify it to suit your microcontroller / compiler.

## 4.6. Use bool data type:

Use bool data type for boolean entities. Do not use int, char, int8_t, uint8_t or any other data type for boolean entities - e.g. flags. Always use bool. If your compiler provides stdbool.h, then include it. Otherwise include stdbool_sedemac.h. This file defines bool data type, and also true and false values (defined as 1 and 0 respectively). Good and bad examples:

bool bFifoFull = false ; // right

uint8_t lifoFull = 0 ;   // not right

## 4.7. Comparing bool variables:

Do not compare bool variables with true or false or anything else. bool variable itself maybe used as a conditional expression.

Good example:

if (bFifoFull)  // recommended

{

// do something

}

if (!bManualMode)   // recommended

{

// not in manual mode

}

Bad example:

if (bFifoFull == true)  // not recommended

{

// do something

}

if (bManualMode == false)   // not recommended

{

// not in manual mode

}

# 5. Legacy Code

## 5.1. Modifying to match this standard:

It is not mandatory to modify entire legacy code to match this standard, since that may require unpractical amount of time and effort.

## 5.2. Maintenance:

This coding standard shall be adhered to for any parts of legacy code that we modify. What is Mandatory, Better and Ideal while maintaining legacy code is listed below.

- Indentation: This standard shall be adhered to for:

  - Lines that we edit. (Mandatory)

  - Code segment that contains the edited lines. (Better)

  - Entire function that contains the edited lines. (Ideal)

- Naming convention: This standard shall be adhered to for:

  - Any new variables and functions that we define (Mandatory)

  - Any existing variables and functions that we use (Better)

  - All variables and functions in the modified module(s) (Ideal).

- If identifier names do not reflect their true meaning and usage:

  - Add a comment near the identifier definition to explain the real meaning and usage of the identifier. You may also mention ideal name for the identifier. (Mandatory)

  - Change the identifier name to reflect it’s true meaning and usage. (Ideal)

- Compiler warnings:

  - There shall be no new compiler warnings generated because of the modifications made in legacy code. (Mandatory)

  - There shall be no compiler warnings generated by the functions that were modified. (Better)

  - There shall be no compiler warnings in entire code. (Ideal)

8. Stakeholders:

| Name of Function/Division/Sub-division, External Customer | Endorsement Available? |
| --- | --- |
| Software Development Engineers | Yes |

| Prepared By: Pramod Ranade Chief Engineer- Software | Reviewed By: Pratibha Walse Dy. Manager- QMS | Approved By: Amit Dixit CTO |
| --- | --- | --- |